|  | ECOVISION IST – 2001 - 32114 Artificial vision systems based on early-cognitive cortical processing |
| --- | --- |

23rd of May 2004

ECOVISION deliverable D3.3
Cross Modal interactions between the Gestalts define the first level of recursive scene analysis

# Introduction

In previous reports, several cross-modal interactions have been formulated: Multi-modal stereo, 'forward' stereo, stereo and grouping, stereo and rigid body motion (RBM). These interactions have already been described to a wide extent in earlier reports. However, they became refined and most importantly quantified in the final phase of the project, using data provided by Riegl UK Ltd.

Figure 1 Image sequence with known ground truth. First row: left and right image for the first frame. Second row: left image fifth frame and ground truth depth data. Those images have been generated from the range data provided by Riegl UK Ltd. (http://www.riegl.com).

As a final step in this project, we are currently working on the interconnecting those process, into a confidence based system. Effectively, we replace binary decision taking in low level process by a confidence evaluation in the decision. The decision itself can then be biased, either statistically by the requirements of the process (high reliability as for RBM estimation, or higher density of estimates), either dynamically by the additional knowledge gathered by another module (e.g. section 3)

Thus, the interconnections in our system become three-fold:

- lateral interactions at the primitive level, through the grouping and interpolation processes,

- bottom-up integration leading to 3D features reconstruction and Rigid Body Motion Estimation,

- and finally top-down feedback, using the computed motion to process ‘forward’ stereo – see section 3.

We will discuss some aspects of this fusion in the following report.

# Use of Point-Line Correspondences for Reconstruction

Although the relative sparseness of the primitives reduces the stereo matching problem, one would expect the definition and the 3D reconstruction accuracy to decrease accordingly.

Indeed, the chance for primitives to be extracted in both images from exactly the same 3D point is very thin. Also, when dealing with edge primitives, the aperture problem prevents us from finding accurate point to point correspondences. Finally, we want to reconstruct the 3D edge defined by those primitives, along with its colour and contrast properties, and not only a position in space.

Actually, the additional information encoded in the primitive (mainly the orientation of the edge) allows us to interpolate information between primitives, along the edges they define and to process reconstruction with sub-pixel accuracy. Consequently, we are matching the position of one primitive of the first image with a whole line (defined by another primitive, via its position and orientation) in the second image.

The plausibility of the reconstructed 3D entity is then used to validate (or invalidate) the proposed stereo match. The cross interaction between planar primitives relations and the plausibility of the resulting spatial hypothesis is a core idea in the following processes.

# “Forward Stereo”: Use of motion for trifocal correction.

If we consider a horizontal edge in the image, then our primitive will have an orientation parallel to the epipolar line. This is unfortunate as all primitives along this edge are bound to be very similar, and due to the aperture problem, the primitive matching becomes completely ambiguous. Similarly, as edges become more horizontal, the stereo matching of the primitives extracted from those edges will be more ambiguous, and also the 3D reconstruction from the stereo pair will become more unstable − as the 3D plane and line becomes progressively coplanar, cf. section 2 and Figure 2 A and B.

Figure 2 A: Stable reconstruction of 3D orientation in case of large difference of orientation of 2D−primitives and epipolar line. B: Unstable reconstruction of 3D orientation in case of similar orientation of 2D−primitives and epipolar line. C: Epipolar lines in case of forward motion for two points. D: Reprojection of 3D−primitives for left/right stereo. Note the missing information for the horizontal structures. E: Reprojection of 3D−primitives for forwad stereo.

For example, in the image shown in Figure 2 D the horizontal structure over the street can not been reconstructed since it has basically the same orientation than the epipolar line. However, it represents important information.

If we assume the motion of the object on which lies the edge, then we can compute a stereopsis between one camera frame, and a later one. The interesting property of this new stereo, is that ego-motion is rarely purely lateral. Consequently, in the general case, the epipolar geometry of this second stereo will be different than the one of the classical stereo, and, what is especially interesting for our problem, the epipolar lines will not be horizontal. For example, if one consider a forward motion (probably the most common case of ego−motion) then the epipolar lines are radial, intersecting at the heading point − see Figure 2 C.

In only very few cases, the temporal and classical epipolar lines will be parallel. This means that using this alternate stereo enables us to recover all of the other horizontal structures of the images − see Figure 2 E.

This is an example of feedback from higher levels of processing to the primitive level: A first set of 3D hypotheses is generated from matching the extracted primitives. From those correspondences the motion is estimated and this motion estimate itself enables to disambiguate the stereopsis of other parts of the same object.

# Stereo and Rigid Body Motion

We use a spatial-temporal constraint based on rigid motion. Assuming the ego-motion or the motion of objects between frames is known we can predict the occurrence of spatial primitives in the next frame. The validation of such correspondences is an indicator for a higher likelihood for the spatial primitive to be a correct one and the associated confidence becomes increased − see also [4, 7]. This scheme has already been described in an earlier report. However, in the last months we did significant improvements by modifying parameters in the comparison and reconstruction of visual entities. With our new ground truth data we are now also able to quantify the performance of our schemes.

Figure 3 Accumulation: this shows the accumulated features after 5 iterations of the scheme. Confidences associated to different 3D primitives are displayed as well as the full set of hypotheses. As can be seen many hypotheses are actually wrong and have low confidences (dark line segments) associated.

Table 1 (third column in each sub-part) gives quantitative results after 3, 5 and 8 iterations. As can be seen from the results even for quite unreliable stereo based on one modality only after only few iterations the number of false positives can be decreased significantly. Note that the scheme allows for the integration of new hypotheses generated in new frames.

# Grouping and Motion Estimation

As a final step in the project we want to combine grouping with RBM estimation. We will use grouping to approach the RBM estimation problem in terms of three aspects:

Reduction and Reliability: Since local features become grouped to larger entities we can reduce the space of possible correspondences considerably − see Figure 4a. Groups of entities have also higher reliability than single entities since they are confirmed by their context. For pose estimation we can start in a natural way with correspondences of the more reliable, larger groups.

Figure 4 a) For each entity in the top row there are 6 correspondences, possibly leading to 66 =46,656 possible patterns of correspondences. Grouping leads to a reduction to 22=4 correspondences only. b) The rich semantic of the image descriptors can be used to neglect correspondences with no fitting attributes. For example, without the colour attribute there are 7 possible correspondences. That can be more than halved by using colour. c) Local estimates of position and orientation can be quite distorted (especially in case of low contrast structure). Grouping can improve the accuracy of such local estimates, resulting in a better estimate of the motion.

- Strong Constraints: The number of necessary correspondences to compute one pose is much smaller if local features become combined into groups since each of the features realizes one constraint for pose estimation. If, for example, a group is constituted by a corner point and the three lines intersecting in this point one correspondence is sufficient while in case of a single point correspondence three correspondences are needed.

- Higher Accuracy: Within a group semantic properties of entities can be estimated with higher accuracy. For example, the orientation and position of a line can be interpolated by taking a number of points into account − see Figure 4c. Based on the improved features, we will then also achieve more precise pose estimates.

We are currently in a testing phase and hope to be able to provide results by the ending of the funding period.

# Quantitative results

As stated previously, using the colour range data provided by Riegl UK Ltd., we have been able to quantify accurately the performance of the different modules of our system. In Table 1, we show the performance of the system on a sequence of images with known ground truth − shown in Figure 1. The results for a stereo with only one modality (orientation), two modalities (orientation and phase)  and three modalities (orientation, phase and colour) respectively are displayed in the left part, middle part and right part in the first column. Here, we would like to remark that the results for colour need to be regarded carefully. We created from one image with known ground truth the other stereo image and the images of consecutive frames assuming a certain motion. Consequently, the colour of corresponding pixels in both images is exactly similar, leading to an unrealistically large performance of the colour cue, compared to the more realistic, less ideal conditions, of real video sequences.

| True | Uni-modal (orientation only) |  |  | Bi-modal (orientation and phase) |  |  | Tri-modal (orientation, phase, colour) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stereo | +grouping | +accu (it. 3/5/8) | Stereo | +grouping | +accu (it. 3/5/8) | Stereo | +grouping | +accu (it. 3/5/8) |
| 50 | 1121 | 819 | 7 / 5 / 4 | 45 | 32 | 1 / 5 / 7 | 0 | 1 | 0 / 2 / 0 |
| 100 | 1479 | 1064 | 12 / 8 / 7 | 77 | 60 | 3 / 6 / 8 | 5 | 4 | 1 / 5 / 4 |
| 500 | 2126 | 1600 | 34 / 32 / 23 | 346 | 262 | 8 / 11 / 14 | 19 | 24 | 9 / 16 / 11 |
| 1000 | 2878 | n.a. | 237 / 102 / 55 | 832 | 586 | 37 / 25 / 16 | 85 | 78 | 22 / 19 / 16 |
| 2000 | n.a. | n.a. | 4784 / 1372 / 402 | n.a. | n.a. | 332 / 153 / 69 | 328 | 278 | 73 / 42 / 27 |

Table 1 This table shows the number of false positives depending on three fixed numbers of correct correspondences is shown for stereo, stereo+grouping, and stereo+grouping+accumulation. The results are given when using one, two and three modalities. The following method has been used for gathering those results: the thresholds have been tuned until reaching a set number of accurate correspondences (50, 100, 500, 1000 and 2000 shown in the first column). Then for this value, the number of false positives is counted, and reported in the other columns. Those are grouped in 3 sections, holding the results when using one, two or three modalities in the processes. Those sections themselves are split in three columns, the first giving the result for the standard stereopsis. The second column shows the false positives when combining stereo with grouping, and the last one when the accumulation is used. This last column shows the results after 3, 5 and 8 iterations of the accumulation scheme. In this array, ‘n.a.’ means that this particular method never reached the number of correct correspondences stated in the first column.  Those results have been obtained using Reigl range data − see Figure 1.

# Conclusion

As discussed above, all processes mentioned in section WP3.3 are working on the basis of modification of confidences associated to hypotheses. In the multi-modal primitives binary decisions transfer the continuous space of filter responses to a discrete space of symbol-like structures represented by our primitives. However, such decisions can not be made at this level with full certainty since local visual information is necessarily ambiguous − see, e.g., [1, 7]. There are two ways to soften the effect of the hard decision in the sparsification: (1) using a soft, confidence based decision process and (2) keeping memory of multiple − and sometimes conflicting − hypotheses for later disambiguation.

Both strategies are used in our approach: to each primitive a confidence is associated that reflects the reliability of the feature attributes. In this sense, the primitives are designed as first guesses with associated confidences that are not expected to deliver completely reliable information but become stabilized by the spatial and spatial-temporal context − see section WP3.3 and [4, 7]).

The meaningfulness of the orientation, phase and colour interpretation of the primitive depends essentially on the local structure. For example, orientation is ill defined for a homogeneous image patch or a corner. However, it makes perfect sense for an edge− or line−like structure. To associate a confidence to the orientation we measure the 'edges-ness' of the local signal by a continuous concept for homogeneousness, edge-ness and corner-ness in terms of intrinsic dimensionality − see [2, 5]. Similarly, the confidences for phase and colour depend on the intrinsic dimensionality. Also, the reliability of optic flow algorithms relative to intrinsic dimension has been analyzed in [3].

References

- Aloimonos, Y. & Shulman, D. (1989). Integration of Visual Modules – An extension of the Marr Paradigm. Academic Press, London, 1989.

- Felsberg, M., & Krüger, N. (2003). A probabilistic definition of intrinsic dimensionality for images. Pattern Recognition, 24th DAGM Symposium, 2003.

- Kalkan, S., Calow, D., Felsberg, M., Wörgötter, F., Lappe, M., and Krüger, N. (2004). Optic flow statistics and intrinsic dimensionality. Brain Inspired Cognitive Systems, Stirling, UK, 2004.

- Krüger, N., Ackerman, M. & Sommer, G. (2002). Accumulation of object representations utilizing interaction of robot action and perception. Knowledge Based Systems, 15:111-118, 2002.

- Krüger, N. & Felsberg, M. (2003). A continuous formulation of intrinsic dimension. Proceedings of the British Machine Vision Conference, 261-270, 2003.

- Krüger, N., & Felsberg, M. (2004). An explicit and compact coding of geometric and structural information applied to stereomatching. Pattern Recognition Letters, 25(8):849-863, 2004.

- Krüger, N., & Wörgötter, F. (2004). Statistical and deterministic regularities: Utilization of motion and grouping in biological and artificial visual systems. Advances in Imaging and Electron Physics, 131, 2004.

- Pugeault, N. & Krüger, N. (2003). Multi--modal Matching applied to Stereo. Proceedings of the British Machine Vision Conference, 2003.

- Pugeault, N., Wörgötter, F. & Krüger, N. (2004). A non-local Stereo Similarity based on Collinear Groups. Proceedings of the Fourth International ICSC Symposium on ENGINEERING OF INTELLIGENT SYSTEMS, 2004.